SBC-IMX8MM

Quick Operation Guide

EMTOP Technology Co.,Ltd.

www.emtop-tech.com

Table of Contents

Product Overview	1

1. Packing List	1

2. Factory Default Conditions	1

Preparations	2

1. Setting Up HyperTerminal	2

2. Hardware Connects	6

Linux Operating System	7

1. Create System TF Card	7

2. Update of TF Card System Image	8

3. Update eMMC	9

Technical Support and Warranty	10

Product Overview

Packing List

SBC-8MX8MM×1

Serial Cable (USB To TTL)×1

12V Power Adapter×1

Cross-Over Network Cable×1

- MIPI To HDMI_LVDS Adapter Board X1

Factory Default Conditions

A Linux system is already programmed in the on-board EMMC and supports HDMI by default; If another display mode is required, please refer to the Display Mode Configurations in user manual.

SBC-IMX8MM has no CR1220 battery installed on board; Please purchase it if necessary.

SBC-IMX8MM Single Board Computer preferably boots from EMMC by default, if you want to boot from TF Card, you need to press Boot Button before Power up

- Wiki

Preparations

- Setting Up HyperTerminal

Click Start > Programs > Accessories > Communications > HyperTerminal on your PC’s desktop;

Start HyperTerminal

Enter a name in the Name field and select an icon below, and then click OK;

Enter a Name

Select the port to be used and click OK;

Select Port

Please configure the port you selected as shown in the following window, and then click OK;

Configure Port

HyperTerminal has been set up successfully.

HyperTerminal Window

Hardware Connects

Hardware Connections

Connection Details

| No. | Descriptions | No. | Descriptions |
| --- | --- | --- | --- |
| 1 | Connect Serial Cable | 4 | Connect a Power Adapter |
| 2-3 | Connect HDMI Adapter Board | 5-6 | If you want to boot from TF card, Need Press boot button before Power Up |

Linux Operating System

- Create System TF Card

- Download HP USB Disk Storage Format Tool 2.0.6 , and use it to format TF card; the figure shown below is the tool’s interface;

Format TF Card

Select FAT32 in the File system drop-down menu, and then click Start to format TF card.

Note:

It is not recommended to use other versions of HP USB Disk Storage Format Tool.

HP USB Disk Storage Format Tool will erase the partitions of TF card.

IMX8MMINI DEMO can boot up from both TF card and eMMC, this section briefly introduce the process of system update on TF card and eMMC.

Update of TF Card System Image

  - Make A Bootable TF Card

- Get the system image from Image directory, named as imx-image-xxx.img.xz, unxz it and create a raw image imx-image-xxx.img.

- If you work under Windows system, please run Tools/win32diskimager to write the imx-image-xxx.img into TF Card. If you work under Linux system, please use dd command to write it into TF Card.

| Image Name | Display Supported |
| --- | --- |
| imx-image-full-imx8mmddr4evk-xxx.img | MIPI-DSI |
| imx-image-full-imx8mmddr4evk-xxx-hdmi.img | HDMI |

Note:

- The difference between imx-image-full-imx8mmddr4evk-xxx.img and imx-image-full-imx8mmddr4evk-xxx-hdmi.img is that the fdt_file in uEnv.txt chooses a different dtb.

- Update U-Boot

If you’ve made some changes to the u-boot source code, and want to update it into TFCard, please run the below command:

- dd if=<YOUR_PATH>/flash.bin of=/dev/sdx bs=1K seek=33 conv=notrunc

Note:

- /dev/sdx is the TFCard device node recognized under Ubuntu system.

- Update Kernel

If you have modified the kernel source code, please update the dtb and Image under Partition 1 [FAT32] of the TF Card. That partition can be recognized by Windows or Linux.

- Update Rootfs

Because EXT4 isn’t accessible Under Windows, please mount the Partiton 2 of TF Card under Ubuntu, change the target file and umount the Card.

Note:

- Press down the button "BOOT" before power up, don't release it. Wait until booting message shows on the terminal [3 seconds at most], release the button.

Update eMMC

  - Make a bootable TFCard and boot up the system;

  - Choose the target image [under directory Image/] and copy it into the USB disk. If it is .xz file, please unxz it to generate .img file.

  - Install the USB disk on the ARM board, it will be automatically mounted under directory /run/media/, for example, the USB disk is recognized as sda1;

  - Run command to start writing eMMC:

- root@imx8mmddr4evk:~# umount /dev/mmcblk2*

- root@imx8mmddr4evk:~# dd if=/run/media/sda1/imx-image-full-imx8mmddr4evk-xxx.img of=/dev/mmcblk2

After it's done, power off the board, remove the TFCard, then reboot the board, it should boot from eMMC and enter into Linux prompt.

Technical Support and Warranty

Technical Support

Emtop Technology provides its product with one-year free technical support including:

Providing software and hardware resources related to the embedded products of Emtop Technology;

Helping customers properly compile and run the source code provided by Emtop Technology;

Providing technical support service if the embedded hardware products do not function properly under the circumstances that customers operate according to the instructions in the documents provided by Emtop Technology;

Helping customers troubleshoot the products.

The following conditions will not be covered by our technical support service. We will take appropriate measures accordingly:

Customers encounter issues related to software or hardware during their development process;

Customers encounter issues caused by any unauthorized alter to the embedded operating system;

Customers encounter issues related to their own applications;

Customers encounter issues caused by any unauthorized alter to the source code provided by Emtop Technology;

Warranty Conditions

- 12-month free warranty on the PCB under normal conditions of use since the sales of the product;

The following conditions are not covered by free services; Emtop Technology will charge accordingly:

Customers fail to provide valid purchase vouchers or the product identification tag is damaged, unreadable, altered or inconsistent with the products.

Products are damaged caused by operations inconsistent with the user manual;

Products are damaged in appearance or function caused by natural disasters (flood, fire, earthquake, lightning strike or typhoon) or natural aging of components or other force majeure;

Products are damaged in appearance or function caused by power failure, external forces, water, animals or foreign materials;

Products malfunction caused by disassembly or alter of components by customers or, products disassembled or repaired by persons or organizations unauthorized by Emtop Technology, or altered in factory specifications, or configured or expanded with the components that are not provided or recognized by Emtop Technology and the resulted damage in appearance or function;

Product failures caused by the software or system installed by customers or inappropriate settings of software or computer viruses;

Products purchased from unauthorized sales;

Warranty (including verbal and written) that is not made by Emtop Technology and not included in the scope of our warranty should be fulfilled by the party who committed. Emtop Technology has no any responsibility;

Within the period of warranty, the freight for sending products from customers to Emtop Technology should be paid by customers; the freight from emtop to customers should be paid by us. The freight in any direction occurs after warranty period should be paid by customers.

Please contact technical support if there is any repair request.

Note:

Emtop Technology will not take any responsibility on the products sent back without the permission of the company.

Contact Information

Technical Support

Tel:		+86-0755-23742193

Email:	support@emtop-tech.com

Sales Information

Email: 	sales@emtop-tech.com

Company Information

Website:	 www.emtop-tech.com